Adama Science and Technology University

School of Electrical Engineering and Computing

Electrical Power and Control Engineering

Final Exam for Electrical Engineering and Computing

2nd Year students

| Course Name: | Fundamentals of Electrical Engineering | Course code: | EPCE2101 |
| --- | --- | --- | --- |
| Year: | II | Time Allotted: | 3:00 Hrs |
|  |  | Date of Exam: | Apr 13th, 2022 |

Name: _______________________________________________

I.D: _____________________

Group: _________

Instructions!!

- Make sure that the Exam has 3 Parts and 13 Questions.

- Possession of any kind of Cellphone/Tablet during exam is strictly forbidden.

- Solutions/Answers to questions should be clear and to the point.

- Unclear/Unreadable answers will be considered as wrong answers.

For Instructors Use Only!

| Question Category | Part-I (6%) | Part-II (10%) |  |  |  | Part-III (20%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Question # |  | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | Total |
| Marks Allotted | 6 % | 2.5% | 2% | 2.5% | 3% | 6% | 6% | 6% | 6% | 40% |
| Marks Obtained |  |  |  |  |  |  |  |  |  |  |

Good Luck!!

PART I. Choose the BEST answer among the given alternatives [6 Pts]

- The Thevenin resistance at terminals a and b is:

| - 5 Ω | - 25 Ω | - 4 Ω | - 15 Ω |
| --- | --- | --- | --- |

- When the voltage across a Capacitor is doubled, the energy stored is:

| - The same | - Quadrupled | - Halved | - Doubled |
| --- | --- | --- | --- |

- If v1=30Sin(ωt + 10°) and v2=-20Cos(ωt+50°), which of these statements is true?

| - v1 leads v2 by 130° | - v1 and v2 are in phase |
| --- | --- |
| - v1 lags v2 by 130° | - None |

PART II. Answer the following questions precisely [10 Pts]

- What is Superposition Principle? [2.5 Pts]

- What is Step Response? [2 Pts]

- Define the following terms? [2.5 Pts]

| - Impedance | - Admittance | - Conductance |
| --- | --- | --- |
| - Reactance | - Susceptance |  |

- Plot the phasor diagrams of voltage and current in: [3 Pts]

| - Resistors | - Capacitors | - Inductors |
| --- | --- | --- |

PART III. Workout the following questions by showing all the necessary steps [24 Pts]

- For the following linear circuit below. [6 Pts]

- Calculate the current io?

- What value of input voltage is necessary to make io = 5A?

- For the circuit shown below: [6 Pts]

- Find the Thevenin equivalent circuit with respect to terminals a-b?

- Find the Norton equivalent circuit using Source transformation?

- If RL= 6Ω then find the voltage drop across RL?

- Find the value of RL for maximum power transfer?

- Find the maximum power transferred?

- From an Automobile Ignition System, we consider the voltage generating system. The system is modeled by the circuit shown in the figure below. The 12-V source is due to the battery and alternator. The 4-Ω resistor represents the resistance of the wiring. The ignition coil is modeled by the 8-mH inductor. The 1-μF capacitor (known as the condenser to auto-mechanics) is in parallel with the switch (known as the breaking points or electronic ignition which opens at t=0s). [6 Pts]

Thus,

- Determine the voltage drop VL(t) across the Ignition coil to create spark.

- Determine the voltage drop across the Capacitor VC(t) and Resistor model VR(t)

- For the RL circuit given below: [6 Pts]

- Find the Total impedance (ZT) of the circuit?

- Find the Total Current (IT) of the circuit?

- Find the voltage drop across the Inductor VL(t) and Resistor VR(t)?

- Plot the Impedance phasor diagram (ZT, ZR and ZC)

- Plot the Phasor diagram for Supply voltage (VS), and the Total current (IT)