Adama Science and Technology University

School of Electrical Engineering and Computing

Department of Electrical Power and Control Engineering

Fundamentals of Electrical Engineering (EPCE210)

Chapter Nine Handout On

Frequency Response

Prepared by

1. Mr. Adisu S. (M.Sc.)

2. Mr. Eyasu Birhanu (M.Sc.)

Reviewed by

1. Mr.Raji A. (Msc)

2. Mr.Getachew T. (Msc)

# CHAPTER NINE

# FREQUENCY RESPONSE

In our sinusoidal circuit analysis, we have learned how to find voltages and currents in a circuit with a constant frequency source. If we let the amplitude of the sinusoidal source remain constant and vary the frequency, we obtain the circuit’s frequency response. The frequency response may be regarded as a complete description of the sinusoidal steady-state behavior of a circuit as a function of frequency. Moreover, the frequency response of a circuit is the variation in its behavior with change in signal frequency or the variation of the gain and phase with frequency of the circuit.

The sinusoidal steady-state frequency responses of circuits are of significance in many applications, especially in communications and control systems. A specific application is in electric filters that block out or eliminate signals with unwanted frequencies and pass signals of the desired frequencies. Filters are used in radio, TV, and telephone systems to separate one broadcast frequency from another.

## 9.2. TRANSFER FUNCTION

The transfer function H(ω) (also called the network function) is a useful analytical tool for finding the frequency response of a circuit. In fact, the frequency response of a circuit is the plot of the circuit’s transfer function H(ω) versus ω, with ω varying from ω = 0 to ω =∞.

The transfer function H(ω) of a circuit is the frequency-dependent ratio of a phasor output Y(ω) (an element voltage or current) to a phasor input X(ω) (source voltage or current).

In general, a linear network can be represented by the block diagram shown in below.

Fig 9.1. A block diagram representation of a linear network.

Thus, by assuming zero initial condition, Since the input and output can be either voltage or current at any place in the circuit, there are four possible transfer functions:

Consider the following series RC shown figure below,

Series RC circuit

The transfer function of voltage gain given by

The magnitude and phase of voltage gain is given by

where ω0 = 1/RC. The frequency the system seen as frequency vary starting from 0 to infinity as shown in the table below.

Fig. 9.3. Amplitude response

Fig. 9.4. Amplitude response

As seen from diagram at ω = 0, H = 1 and φ = 0. At ω = ∞, H = 0 and φ = −90◦. Also, at ω = ω0, H = 1/ √2 and φ = −45◦

## 3. Resonance

The resonant circuit is a combination of R, L, and C elements having a frequency response characteristic similar to the one appearing in Fig. below. Resonant circuits (series or parallel) are useful for constructing filters, as their transfer functions can be highly frequency selective. They are used in many applications such as selecting the desired stations in radio and TV receivers.

Fig. 9.5 resonance curve

The response is maximum for the frequency fr, decreasing to the right and left of this frequency. In other words, for a particular range of frequencies the response will be near or equal to the maximum. The frequencies to the far left or right have very low voltage or current levels and, for all practical purposes, have little effect on the system’s response. The radio or television receiver has a response curve for each broadcast station of the type indicated in figure above. When the receiver is set (or tuned) to a particular station, it is set on or near the frequency fr shown above. Stations transmitting at frequencies to the far right or left of this resonant frequency are not carried through with significant power to affect the program of interest. The tuning process (setting the dial to fr) as described above is the reason for the terminology tuned circuit. When the response is at or near the maximum, the circuit is said to be in a state of resonance.

The resonant electrical circuit must have both inductance and capacitance. In addition, resistance will always be present due either to the lack of ideal elements or to the control offered on the shape of the resonance curve. When resonance occurs due to the application of the proper frequency (fr), the energy absorbed by one reactive element is the same as that released by another reactive element within the system. In other words, energy pulsates from one reactive element to the other.

Therefore, once an ideal (pure C, L) system has reached a state of resonance, it requires no further reactive power since it is self-sustaining. In a practical circuit, there is some resistance associated with the reactive elements that will result in the eventual “damping” of the oscillations between reactive elements. A resonant circuit is characterized by five related parameters: the two half-power frequencies ω1 and ω2, the resonant frequency ω0, the bandwidth B, and the quality factor Q.

There are two types of resonant circuits: series and parallel. Each will be considered in some detail in this chapter.

3.1. series resonance analysis

Consider the series RLC circuit shown in figure below in the frequency domain.

Fig. 9.5 Series resonant circuit

The input impedance is

Resonance results when the imaginary part of the transfer function is zero, or

The value of ω that satisfies this condition is called the resonant frequency ω0. Thus, the resonance condition is

Thus,

Note that at resonance:

- The impedance is purely resistive, thus, Z = R. In other words, the LC series combination acts like a short circuit, and the entire voltage is across R.

- The voltage Vs and the current I are in phase, so that the power factor is unity.

- The magnitude of the transfer function H(ω) = Z(ω) is minimum.

- The inductor voltage and capacitor voltage can be much more than the source voltage

The highest power dissipated occurs at resonance, when I = Vm/R, so that

At certain frequencies ω = ω1, ω2, the dissipated power is half the maximum value; that is,

Hence, ω1 and ω2 are called the half-power frequencies.

The half-power frequencies are obtained by setting Z equal to √2R, and writing

Solving for ω, we obtain

The width of the response curve depends on the bandwidth B, which is defined as the difference between the two half-power frequencies and it is the width of the frequency band between the half-power frequencies.

The “sharpness” of the resonance in a resonant circuit is measured quantitatively by the quality factor Q. The quality factor relates the maximum or peak energy stored to the energy dissipated

in the circuit per cycle of oscillation:

In the series RLC circuit, it is given by

Or

The relationship between the bandwidth B and the quality factor Q is given by

Or

Thus, the quality factor of a resonant circuit is the ratio of its resonant frequency to its bandwidth. the higher the value of Q, the more selective the circuit is but the smaller the bandwidth. The selectivity of an RLC circuit is the ability of the circuit to respond to a certain frequency and discriminate against all other frequencies. If the band of frequencies to be selected or rejected is narrow, the quality factor of the resonant circuit must be high. If the band of frequencies is wide, the quality factor must be low. A resonant circuit is designed to operate at or near its resonant frequency.

The higher the circuit Q, the smaller the bandwidth.